Direct Memory Access using CDMA

In the Zynq, multiple interconnections are available between the PS and PL sections with different performance levels for data transfer between the two subsystems. The General Purpose (GP) Master and Slave AXI interconnect are intended for peripherals that do not have high bandwidth requirements. There are four High Performance PS slave to PL master AXI interfaces available for peripherals that need higher bandwidth such as video and image processing applications. In this demo, we will enable a High Performance AXI slave port in the PS, add an AXI central DMA (CDMA) controller, and perform Direct Memory Access (DMA) operations between various memories.

Design Description

In this demo, we will enable the HP port of the PS and add an instance of the Central DMA (CDMA) controller in the PL.  We will add two instances of an AXI-BRAM controller, one will access the second port of the BRAM via the processor.  We will connect the interrupt request line from the CDMA to the input of the GIC of the PS.  The following diagram represents the completed design (Figure 1).

Figure 1. Completed Design

Create the Project	Step 1

Open the Vivado program. Create a project with GPIO and one BRAM.

Start Vivado.

Create a block diagram with a Zynq processing system and select Run Block Automation.

Double-click on the Zynq processing system instance to open its configuration form.

Click on the MIO configuration panel to open its configuration.  Expand the I/O Peripherals (and GPIO).  Deselect ENET 0, USB 0, SD 0, and expand GPIO and deselect USB Reset and I2C Reset.  These are things we are not going to use, so we can disable them to keep the Zynq system simpler.

Create 2 AXI GPIO ports, one for switches and one for LEDs.  Use AXI_GPIO_0 for the switches and AXI_GPIO_1 for the LEDs.  Connect them to the board interface.  Run Connection Automation selecting both GPIO components. The block design is shown below.

- Figure 2. Initial Block Design with GPIO

Configure the Processor to Enable M_AXI_GP1

Open the Block Design and enable the M_AXI_GP1 interface.

Double-click on the Zynq processing system instance to open its configuration form.

Select PS-PL Configuration in the Page Navigator window in the left pane, expand AXI Non Secure Enablement>GP Master AXI Interface, and click on the check-box of the M_AXI GP1 Interface to enable it.

Select Clock Configuration in the Page Navigator window in the left pane, expand PL Fabric Clocks on the right, and click on the check-box of the FCLK_CLK1 to enable it.

Enter the Requested Frequency for the FCLK_CLK1 as 140.00000 MHz.

Click OK to accept the settings and close the configuration form.

Figure 3. M_AXI_GP1 interface enabled

Extend with BRAM	Step 3

Add an AXI BRAM Controller instance with BRAM.

Click the  button and search for BRAM in the catalog.

Double-click the AXI BRAM Controller to add an instance to the design.

Click on Run Connection Automation, and select axi_bram_ctrl_0

Click on BRAM_PORTA and BRAM_PORTB check boxes.

Click S_AXI, and change the Master option to /processing_system7_0/M_AXI_GP1, change the Crossbar clock source of interconnect IP, Clock source for Master interface, and Clock source for Salve interface to /processing_system7_0/FCLK_CLK1 (140 MHz) as they all run in the same clock domain, and click OK

Figure 4. Connecting AXI BRAM Controller to M_AXI_GP1 to run at faster clock speed

Notice that an instance of AXI SmartConnect and Processor System Reset are added, and the M_AXI_GP1_ ACLK is connected to FCLK_CLK1.

Figure 5. Clocking network connections

Double-click on the axi_bram_ctrl_0 instance to open the configuration form.

Set the Data Width to 64.

Figure 6. Setting the BRAM controller data width to 64

Click OK.

Using the Address Editor tab, set the BRAM controller size to 64KB.  Validate the design.

Select the Address Editor tab and notice that the BRAM controller memory space is 8K.

Click in the Range column of the axi_bram_ctrl_0 instance and set the size as 64K.

Figure 7. AXI BRAM space assignment

Notice that the address range changed to 0x80000000-0x8000FFFF.  This is in the M_GP1 addressing space.

Select the Diagram tab, and click on the  (Validate Design) button to make sure that there are no errors.

Configure the Processor to Enable S_AXI_HP0	Step 2

Open the Block Design and enable the S_AXI_HP0 interface

Click the Diagram tab.

Double-click on the Zynq processing system instance to open its configuration form.

Select PS-PL Configuration in the Page Navigator window in the left pane, expand HP Slave AXI Interface on the right, and click on the check-box of the S AXI HP0 Interface to enable it, and click OK to close the Configuration window.

Add CDMA and BRAM	Step 3

Instantiate the AXI central DMA controller.

Click the  button and search for Central in the catalog.

Double-click the AXI Central Direct Memory Access to add an instance to the design.

Double-click on the axi_cdma_0 instance and uncheck the Enable Scatter Gather option.

Change the Write/Read Data Width to 64 and click OK.

Note the burst size changes from 16 to 8. You can increase this up to 256 to improve the performance. Here we are using smallest number since the application allows small number of words transfer.

Run connection automation

- Connection automation could be run on all unconnected ports simultaneously. For the purposes of this demo, each port will be connected separately so that the changes made by the automation process are easier to follow.

Click on Run Connection Automation and select processing_system7_0/S_AXI_HP0 only.

Check that this port will be connected to the /axi_cdma_0/M_AXI port and click OK.

Figure 8. Connection automation

Verify the CDMA connection through the AXI SmartConnect to the HP0 port

Figure 9. Connecting AXI Central DMA controller to S_AXI_HP0

Notice that an instance of AXI Memory Interconnect (axi_mic_1) is added, S_AXI_HP0 of the processing_system7_0 is connected to M00_AXI of the axi_mic_1, S00_AXI of the axi_mic_1 is connected to the m_axi of the axi_cdm_0 instance.  Also, m_axi_aclk of the axi_cdma_0 is connected to the net originating from FCLK_CLK0 of the processing_system7_0.

Click on Run Connection Automation again, and select /axi_cdma_0 (which includes S_AXI_LITE).

Notice that the axi_cdma_0/M_AXI port is no longer available to select. This is because it was connected to the processing system in the previous step.

Ensure /processing_system7_0/M_AXI_GP0 is selected in the drop-down button and click OK.

Figure 10. CDMA connected

Instantiate another BRAM Controller and a BRAM.

Click the  button and search for BRAM in the catalog.

Double-click the AXI BRAM Controller to add an instance to the design.

Click on Run Connection Automation, and select /axi_bram_ctrl_1/S_AXI only.

For the Master connection, select axi_cdma_0/M_AXI from the dropdown box.

Figure 11. BRAM connection automation

Click OK to connect to make the connection.

Notice that another axi interface (M01_AXI) is added to the axi_smc_1 instance and is connected to the S_AXI interface of the axi_bram_ctrl_1 instance.

Figure 12. Connection between the new BRAM controller to the CDMA

Double-click the axi_bram_ctrl_1 instance and change the Number of BRAM Interface to 1.

Change the Data Width to 64 and click OK.

Double-click the axi_bram_ctrl_0 instance and also change the Number of BRAM Interface to 1. Click OK.

Using the wire tool, connect the BRAM_PORTA of the axi_bram_ctrl_1 instance to the BRAM_PORTB of the Block Memory Generator axi_bram_ctrl_0_bram instance.

Figure 13. Connect the second BRAM controller

Connect the CDMA interrupt out port to the port of the processor.

Double-click on the processing_system7_0 instance to open its configuration form.

Select Interrupts in the Page Navigator window in the left pane, check the Fabric Interrupts box.

Expand Fabric Interrupts > PL-PS Interrupts Ports, and click on the check-box of the IRQ_F2P.

Figure 14. Enabling the processor interrupt

Click OK.

Using wiring tool, connect the cdma_introut to the IRQ_F2P port. (Click on the cdma_introut port and drag to the IRQ_F2P port)

Using the Address Editor tab, set the BRAM controller size to 64KB.  Validate the design.

Select the Address Editor tab.

Expand the axi_cdma_0> Data section, and change the memory size of axi_bram_ctrl_1 to 64K.

Figure 15. Address space

The design should look similar to the figure below.

Figure 16. Completed design

Select the Diagram tab, and click on the  (Validate Design) button to make sure that there are no errors.

Create and HDL Wrapper and Generate the Bitstream	Step 4

Create an HDL Wrapper by right-clicking on design_1.bd in the design sources window under the Sources tab and select Create HDL Wrapper.  Click OK to the default selection to let Vivado manage the wrapper.

Click on the Generate Bitstream to run the synthesis, implementation, and bit generation processes.

Click Save to save the project, and Yes if prompted to run the processes. Click OK to launch the runs.

When the bitstream generation process has completed successfully, click Cancel.

Generate an Application in Vitis	Step 5

Export the implemented design, and start Vitis

Export the hardware configuration by clicking File > Export > Export Hardware…

Click the box to Include Bitstream and click OK

Open Vitis

To clean the workspace, right-click on each open project except system_wrapper_hw_platform_2 and select close project.

Create an empty application project, named demoCDMA, and import the provided demoCDMA.c file.

Create a new Workspace by entering workspace_demoCDMA.

Select Create Application Project.

In the Project Name field, enter demoCDMA as the project name, leave all other settings to their default’s and click Next.

For the Platform, select the Create a new platform from Hardware (XSA) tab.  Then click the plus sign and navigate to the directory containing your Vivado project.  Select design_1_wrapper.xsa.  This is the export of your hardware description.  Then click Next.

For the Domain, leave the CPU, OS, Language and boot components as defaulted, click Next.

Select the Empty Application template and click Finish.

Select demoCDMA > src in the project view, right-click, and select Import Sources.

Click Browse next to the From Directory.

Browse to the folder containing the demoCDMA.c file that you downloaded with this demo and click Select Folder.

In the panel on the right, check the box next to demoCDMA.c and click Finish.

In the Explorer tab, select the demo_CDMA_system and click Project > Build Project.

Test in Hardware	Step 6

Connect and power up the board. Download the bitstream and program the FPGA.

Connect and power up the board.

In Vitis, select Xilinx Tools > Program FPGA.

Click the Program button to program the FPGA.

Establish serial communication, and run the demoCDMA application from the DDR3 memory.

Right click on demoCDMA, and select Debug As > Debug Configurations

Double click on Single Application Debug (GDB) to create a new configuration (Debugger_demoCDMA-GDB will be created).

Select the demoCDMA project in Project Explorer, right-click and select Debug As > Debug Configurations.  From the debug configuration options, select Debugger_demoCDMA-GDB that we just created and configured.  Then click Debug to download the application and execute ps7_init. (If prompted, click Yes to switch to the Debug perspective.)  The program execution starts and suspends at the entry point.

Select the  tab.  If it is not visible then select Window > Show view > Terminal.

Click on   and select the appropriate COM port (depending on your computer), and configure it.

Click the resume button in the debug window.  The program will give you options in the Vitis Serial Terminal.

Follow the menu in the terminal emulator window and test transfers between various memories.

In order to send input through the Vitis Serial Terminal, use the input box near the bottom and the Send button.

Figure 17. Vitis Serial Terminal Program Run

Select option 4 in the menu to complete the execution.

Close Vitis and Vivado programs by selecting File > Exit in each program.

Turn OFF the power on the board.

This demo led you through adding a CDMA controller to the PS so that we can perform DMA transfers between various memories. We used the high-performance port so DMA could be done between the BRAM residing in PL and DDR3 connected to the PS. We verified the design functionality by creating an application and executing it from the DDR3 memory.