Profiling and Performance Tuning

This demo guides you through the process of profiling an application and analyzing the output. The application is then accelerated in hardware and profiled again to analyze the performance improvement.

In this demo, we will:

- Setup the board support package (BSP) for profiling an application

- Set the necessary compiler directive on an application to enable profiling

- Setup the profiling parameters

- Profile an application and analyze the output

In this demo, you will design an embedded system that consists of an ARM Cortex-A9 processor SoC and two instances of the provided FIR filter IP. The following diagram represents the completed design (Figure 1).

Figure 1. Completed Design

Create a Vivado Project	Step 1

Launch Vivado and create an empty project, called demoProfile, targeting the Zybo or ZedBoard Zynq Evaluation and Development Kit and using the VHDL language.

Open Vivado and create a new project new project call demoProfile.

Select the RTL Project option in the Project Type form, and click Next.

Select VHDL as the Target Language in the Add Sources form, and click Next.

Click Next two times.

In the Default Part form, click on Boards and select either the Zybo or Zedboard and click Next.

Click Finish to create an empty Vivado project.

Set the project settings to include provided fir_top IP

Click Settings in the Flow Navigator pane.

Expand IP in the left pane of the Project Settings form.

Click Repository and using “minus” button remove entries, if any.

Click on the “plus” button, browse to the directory where you placed the downloaded IP that accompanies this demo and click Select.

Click OK.  The directory will be scanned and it will report one IP was detected.

Click OK twice.

Creating the Hardware System Using IP Integrator	Step 2

Create a block design in the Vivado project using IP Integrator to generate the Zynq ARM Cortex-A9 processor-based hardware system.

In the Flow Navigator, click Create Block Design under IP Integrator.

Name the block system and click OK.

Click on the  button.

Once the IP Catalog is open, type zy into the Search bar, and double click on ZYNQ Processing System entry to add it to the design.

Click Run Block Automation, and click OK to accept the default settings.

Double-click on the Zynq block to open the Customization window for the Zynq processing system.

A block diagram of the Zynq should now be open, showing various configurable blocks of the Processing System.

Configure the I/O Peripherals block to only have UART 1 support.  Deselect the TTC device.

Click on the MIO Configuration panel to open its configuration form.

Expand the I/O Peripherals on the right.

Uncheck ENET 0, USB 0, and SD 0, GPIO (GPIO MIO), leaving UART 1 selected.

In the MIO Configuration panel, expand the Application Processing Unit and uncheck the Timer 0.

Click OK.

Figure 2. ZYNQ Processing System configured block

Add FIR Core to the System	Step 3

Instantiate the provided FIR core twice naming the instances as fir_left and fir_right. Validate the design.

Click the  button and search for fir in the catalog.

Double-click on the fir_top_v1_0 to add the IP instance to the system

Select the fir_top_1 instance and change its name to fir_left in its property form.

Click the  button and search for fir in the catalog.

Double-click on the fir_top_v1_0 to add the IP instance to the system

Select the fir_top_1 instance and change its name to fir_right in its property form.

Click on Run Connection Automation, and select All Automation to select fir_left and fir_right.

Click on s_axi_fir_io for both fir_left and fir_right and confirm that they will be automatically connected to the Zynq M_AXI_GP0 port

Click OK to connect the two blocks to the M_AXI_GP0  interface.

The design should look similar to shown below:

Figure 3. The completed design

It is not necessary to connect the interrupt signals of the fir blocks.

Select the Diagram tab, and click on the  (Validate Design) button to make sure that there are no errors.

Ignore warnings.

Generate the Bitstream	Step 4

Create the top-level HDL of the embedded system, and generate the bitstream.

In Vivado, select the Sources tab, expand the Design Sources, right-click the system.bd and select Create HDL Wrapper and click OK.

Click on the Generate Bitstream in the Flow Navigator pane to synthesize and implement the design, and generate the bitstream.

Click Save to save the design and Yes to run the necessary processes. Click OK to launch the runs.

When the bitstream generation process has completed click Cancel.

Export the Design to Vitis	Step 5

Export the design to Vitis, create the software BSP using the standalone operating system and enable the profiling options.

Export the hardware configuration by clicking File > Export > Export Hardware…

Tick the box to Include Bitstream, and click OK

Launch Vitis.

In Vitis, create an empty application project named demoProfile, and import the provided demoProfile.c file.

Create a new Workspace by entering workspace_demoProfile.

Select Create Application Project.

In the Project Name field, enter demoProfile as the project name, leave all other settings to their default’s and click Next.

For the Platform, select the Create a new platform from Hardware (XSA) tab.  Then click the plus sign and navigate to the directory containing your Vivado project.  Select design_1_wrapper.xsa.  This is the export of your hardware description.  Then click Next.

For the Domain, leave the CPU, OS, Language and boot components as defaulted, click Next.

Select the Empty Application template and click Finish.

Click on Navigate to BSP settings to change the settings in the board support package.

Figure 4. Navigate to BSP Settings

Click Modify BSP Settings

Figure 5. Modify BSP Settings

Notice Standalone_bsp_0 in the Project name field and click Finish with default settings.

A Board Support Package Settings window will appear.

Select the Overview > standalone entry in the left pane, click on the drop-down arrow of the enable_sw_intrusive_profiling Value field and select true.

Figure 6. Enable profiling in the board support package

Select the Overview > drivers > cpu_cortexa9 and add –pg in the extra_compiler_flags Value field.

Figure 7. Adding profiling switch

Click OK to accept the settings and create the BSP.

Create the Application	Step 6

Create the demoProfile application using the provided demoProfile.c, fir.c, fir.h, fir_coef.dat, and xfir_fir_io.h  files.

Select demoProfile > src in the project view, right-click, and select Import Sources.

Click Browse next to the From Directory.

Browse to the folder containing the demoProfile.c file that you downloaded with this demo and click Select Folder.

In the panel on the right, check the box next to demoProfile.c, fir_coef.dat, fir.c, fir.h, and xfir_fir_io.h, and click Finish.

Right click on the demoProfile_system and select Build Project.

Open the demoProfile.c file and scroll to the main function at the bottom. Notice the following code:

The function fir_software( ) function is a software implementation of the FIR function. The filter_hw_accel_input( ) function offloads the FIR function to the two FIR blocks that have been implemented in the PL.

Run the Application and Profile	Step 7

Program the PL section and run the application using the user-defined SW_PROFILE symbol.

Power ON the board.

Select Xilinx Tools > Program FPGA and click on Program.

Right click on the demoProfile Project [standalone on ps7_cortexa9_0], and select C/C++ Build Settings.

Under the ARM v7 gcc compiler group, select the Symbols sub-group, click on the  button to open the value entry form, enter SW_PROFILE, and click OK.

This will allow us to profile the software loop of the FIR application.

Figure 8. Add user-defined symbol

Under the ARM v7 gcc compiler group, select the Profiling sub-group, then check the Enable Profiling box, and click Apply and Close.  If you are asked to rebuild, select Yes to rebuild.

Figure 9. Compiler setting for enabling profiling

Right click on the demoProfile_system and select Build Project.  Once the build is finished, select Xilinx > Program FPGA and click on Program.

Right click on the demoProfile project and Select Run As > Run Configurations… and double click on Single Application Debug GDB to create a new configuration.

Click on the newly created demoProfile Debug configuration, and select the Application tab.

Click on the Advance Options Edit… button.

Click on the Enable Profiling (gprof) check box, enter 100000 (100 kHz) in the Sampling Frequency field, enter 0x10000000 in the scratch memory address field, and click OK.

Figure 10. Profiling options

Click the Run button to download the application and execute it.

The program will run.

Analyze the results.

When execution is completed, the Gmon File Viewer dialog box will appear showing demoProfile.elf as the corresponding binary file.  Click OK.

Click on the Sort samples per function button ().

Click in the %Time column to sort in the descending order.

Note that the fir_software routine is called 60 times, 20 samples were taken during the profiling, and on an average of 3.333 (ZedBoard) or 3.166 (Zybo) microseconds were spent per call.

Figure 11. Sorting results

Go back to the Run Configuration, and change the sampling frequency to 1000000 (1 MHz) and profile the application again.

When execution is completed, click OK and the gprof viewer will be updated.

Invoke gprof, select the Sorts samples per function output, and sort the %Time column.

Notice that the output has better resolution and reports more functions and more samples per function calls. Note that the number of calls to the fir_software function has not changed but the number of samples taken increased, and the average time spent per call is 5.483 microseconds  in the figure below.

Figure 12. Profiled results with 1 MHz sampling frequency

At this stage, the designer of the system would decide if the FIR function should be ported to hardware.

Profile the application using the hardware FIR filter IP by removing the user defined SW_PROFILE symbol.

Select the demoProfile application, right-click, and select C/C++ Build Settings.

Under the ARM v7 gcc compiler group, select the Symbols sub-group, select SW_PROFILE, and delete it by clicking on the delete button.

This will allow us to profile the hardware IP of the FIR application.

Figure 13. Deleting the user-defined symbol

Click Apply, and then click Apply & Close. If prompted to rebuild now, select Yes.

Right click on the demoProfile_system and select Build Project.  Once the build is finished, select Xilinx > Program FPGA and click on Program.

Select Run > Run Configurations and click the Run button to profile the application again and click OK when profiling completes.

- Figure 12. Deleting the user-defined symbol

Close Vitis and Vivado programs by selecting File > Exit in each program.

Turn OFF the power on the board.